Chapter Two

- Failure

  - at the external interface to the system, an event occurs such that the system’s behaviours is not what was required. This bad event is designated as a failure of the system.

- Error

  - A wrong value in the internal state of the system is an erroneous value. The wrong value is designated as an error

- Fault

  - A wrong value in the internal state of a component system is an erroneous value in that components but is designated as a fault in the system.

Principles of Fault Tolerance

- There are technically no rules for fault tolerance techniques due to:

  - Rapidly changing hardware technology/costs

  - Varying dependability requirements

  - Varying design assumptions in building systems

- Overall, “How, where, and how much?”

Principles of Fault Tolerance: How

- Can be split into four phases:

  - Error Detection

  - Damage Confinement and Assessment

  - Error Recovery

  - Fault Treatment and Continued Service

Principles of Fault Tolerance: How: 1. Error Detection

- To provide tolerance, must detect issue in the system

- Unable to detect a fault directly but manifestation of a fault leads to errors somewhere in the internal system state

- Start of fault tolerance is detecting an error when the system is in an erroneous

Principles of Fault Tolerance: How: 2. Damage Confinement and Assessment

- Likely delay between fault manifestation and detection of error, damage may be been caused elsewhere in the system

  - Not everything will have been detected

- May want barriers to the propagation of errors

- May want to assess the extent of the spread of damage

Principles of Fault Tolerance: How: 3. Recovery

- Need to apply error recovery techniques

- These aim to transform the erroneous state into an error-free state

  - So normal system operation can continue

- If the damage introduction by a given fault can always be eradicated by means of error recovery, then that fault cannot cause the system to fail

- A key aspect pf providing fault tolerance

Principles of Fault Tolerance: How: 4. Fault Treatment and Continued Service

- Error recovery may restore the system but the fault will still remain

- It may be necessary to ensure the fault can’t reoccur repeatedly

- Many need to locate the fault

  - Then repair or reconfigure before system can continue normally

Fault Tolerance Principles: How:

- Four phases can form the basis for:

  - The design and implementation of fault tolerance

  - Examining and analysing the fault tolerance provision in an existing system

- Interplay between the phases can blur their identification

  - error detection mechanism

    - assess damage as well as initial detection of errors

  - fault location using diagnostic checks

    - error detection

- Four phases are present and operate sequentially:

  - 1 -> 2 -> 3 -> 4

    - Error Detection

    - Damage Confinement and Assessment

    - Error Recovery

    - Fault Treatment and Continued Service

- Phases 2 & 4 and strongly dependant on structural factors and assumptions concerning the implementation of the system

Redundancy and It’s Classification

- All fault tolerance techniques depend on redundancy

  - Parts of the system or system operation that would not be needed if absence of faults could be guaranteed

- Early redundancy classification distinguished hardware from software

  - Hardware redundancy:

    - Extra circuitry

    - Power supplies

    - Standby hardware units

  - Software Redundancy:

    - Programs which detect problems and deploy hardware redundancy

    - Usually by reconfiguration (switching to a spare memory unit

- Some modern systems contain redundant software utilised for tolerating software (mainly design) faults

- Temporal redundancy:

  - Repeat (a part of) system operation

    - Effective for tolerating transient faults

- Further Classification of Hardware redundancy:

  - Static:

    - Redundant components are used to mask effects of components faults

      - Hence name of ‘Masking Redundancy’

    - Effects of faults are concealed from components environment

    - Error detection is followed by error compensation techniques

    - Phases 2 & 4 are often omitted

      - Error correcting code

  - Dynamic:

    - Redundancy used for error detection only

    - Needs to be supplemented by additional redundancy elsewhere to achieve fault tolerance

    - Examples:

      - Simple parity code:

        - Memory fault would still require redundancy elsewhere, such as operating system code

      - Check sums for message transmissions

Canonical Example of Static Redundancy

- Triple Modular Redundancy (TMR)

- Standard Application of TMR

  - Tolerating hardware component faults

V

C

C

C

- V: Voter

- C: Component

- TMR is effective if:

  - At most one component is faulty

  - And the voter always gets it right

Triple Modular Redundancy

- TMR assumptions are not unreasonable:

  - Voter is a simple comparator device

    - Can be built reliably

  - More than one component fault is unlikely with maintenance

    - Requires component isolation

- Highly critical applications may use NMR

  - Five replicated components (5MR)

    - This is able to tolerate two simultaneous component faults

- TMR is a conceptually simple and highly effective form of fault tolerance, but:

  - Incurs a fixed cost independent of fault frequency

    - Monetary:

      - General purpose computing

    - Space and Weight:

      - Aircraft and space applications

    - Voting time overhead:

      - Real-time applications

  - Only tolerates physical faults, not design faults

- Cheaper, smaller, and faster hardware components makes TMR more feasible

  - Widely used in avionics applications (since 1970’s)

Redundancy Combinations

- Message Transmissions in Distributed System

A

m

B

m’

Single Network

- Networks are normally reliable but can be prone to transient faults

- Possible transmissions problems:

  - Loss of m due to a break, buffer overflow, etc.

    - ie. m’ = null

  - Corruption of m due to noise, buffer corruption, etc.

    - Ie. m’ != m

A

m

B

m’

Single Network

- More reliable message passing service is obtained by:

  - Augmenting m with check sum c (Dynamic redundancy)

  - Retransmitting [m, c] until a positive acknowledgment is returned (temporal redundancy)

- Limitations:

  - Can’t tolerate permanent network partition

  - Varying message delays

    - Depends on the number of retries required

B

A

m

m’

Dual Network

m

m’

Dual Network Redundancy

- Assumptions:

  - At most, one network can develop a fault at any given time

- In this situation, A augments m with checksum c and transmits [m,c] in parallel on both networks

  - A does not now wait for a positive acknowledgement since message loss/corruption along one path will be masked by dual (static) redundancy

B

A

m

m’

Dual Network

m

m’

- Using the checksum, B can distinguish between correct and corrupt m’

- The algorithm for B to receive a message is:

  - Accept first arriving and error free m’

  - Discard/ignore duplicate or erroneous m’

- Adding temporal redundancy:

  - This provides tolerance to permanent or transient failure in one network and transient failure in the other

Additional Example

actuators

Voter

A2

A1

sensors

Failure free Network

- Failure Free Network:

  - Various network designs with various network faults assumptions

- Fault Assumptions

  - Either A1 or A2 (but not both) can suffer a transient fault in a given round, causing no or incorrect results to be sent

  - Voter never fails and can detect a processor’s null response

- Dual computation

  - Voter receives sensor input; requests A1 and A2 to perform the required computation

  - A1 and A2 do so in parallel, and send the results back

  - Voter collects the results and produces its output according to some algorithm

  - It can ask for a re-computation fi the first round is not satisfactory

Failure Free Network: Voter Algorithm

get-results (a1, a2);
if a1 = a2 then {
    output a1; exit
}
if a1 = null || a2 = null then {
    output non-null value;
    exit
}
if a1 != a2 then {        // this must be the case
    get-results (a1, a2);        // ask for re-computation
    if a1 = a2 then {
        output a1; 
        exit
    }
    if a1 = null || a2 = null then {
        output non-null value;
        exit
    }
    if a1 != a2 then {
        raise alarm and bring environment to a safe state;
        exit;
    }
}

Dependability Evaluation

- Redundancy can increase system dependability, but can also increase cost and complexity

- Fault tolerant systems should have evaluation performed

  - Inform designer

  - Convince customer

- Dependability evaluation is research

- Start at Hardware reliability evaluation

  - Software operation depends on hardware reliability

  - Component failure rates can be assessed and predicted

- Using testing to work out failure rates of hardware

- Reliability of hardware component is modelled as:

  - R(t) = exp(-~t)

    - ~ is the sum of the failure rates of all sub components and the mean time between failures

    - MTBF = 1/~

- Assumptions commonly made in the reliability evaluation of hardware systems

  - Component fail independently

  - The testing environment accurately models the operational environment

  - The behaviour of a physical component is predictable from data obtained during testing of another component of the same type

  - The system being evaluated has no design faults

Dependability Evaluation: Design Faults

- Design faults and their erroneous effects cannot be anticipated

- Unlike physical faults, eradication of design faults is permanent

  - If we had accurate knowledge about design faults we would remove them

- Nevertheless, there are methods for predicating software failure rates

  - Most methods are based on monitoring the improvement in dependability as testing progresses (“reliability growth models”)

  - These methods try to take into account the likelihood of encountering a fault in the software (“bug sizes”)

Where to Deploy Fault Tolerance

- Designing a fault tolerant system is a n iterative process

- Aim to minimise redundancy and maximise dependability, subject to cost constraints

  - A trade off

- Available alternative follow from the system model:

System

Design

Components

Sub-Design

Sub-Components

- System = components + design

- dependability can only be compromised by component faults or design faults

- Component faults can result from sub-design or sub-component faults

  - Within the component

- And so on

  - Until atomic components are reached

- Thus, options open to the system designer for the placement of redundancy relate to the “level” at which a fault is identified

- To tolerate design faults, must use redundancy within the system

- To tolerate component faults, could similarly use redundancy within the system

  - Or……………

- Could seek to enhance the dependability of the components:

  - Examining their internal structure and incorporating some internal redundancy

  - Appropriate for “fault prone” sub-components

- Hardware systems have “low level” redundancy and it is very economic and effective

  - But only for specific faults

- In contrast, redundancy at the system level to try to deal with (the effects of) component failures may be more costly

  - But may be effective for a broader range of faults

- Example: semiconductor memory

  - Single bit errors are common and Error Correcting Codes (ECC) provide tolerance at an additional cost of approximately 25%

  - Triplication incurs an additional 200% cost, but would guard against multiple-bit errors, address decoding errors, etc.

A Software Design Framework using Dynamic Redundancy

- Dynamic redundancy only detects errors

  - Additional redundancy will still be required to implement the other phases of fault tolerance

  - Indeed, different forms of redundancy server different purposes

- Incorporating redundancy must itself result in some increase in system complexity

  - To minimise the risk from that increase in complexity, the provisions for fault tolerance should be carefully structured

  - Therefore, propose an implementation framework for fault tolerance which clearly separates normal and abnormal (fault tolerance) activities

Modular Programming Approach

- Assume a structured approach to software development

- Consider a software system organised as a hierarchy of modules

  - An arrow from module A to module B  means that A is a user of B

  - Hence satisfactory completion of some operations by A will depend on satisfactory services being provided by B

A

C

B

E

D

Resilient Modules

- Consider the task of turning B into a more “resilient” module

- In the absence of faults:

Module B

Correct Service Requests

Correct Service Requests

Correct Responses

Correct Responses

- What happens if shit goes fucked up?

  - Impossible to state (no details, no provision)

- What if the modules have built in error detection capability?

  - Then a module can respond differently if something wrong is detected within the module or in the received requests or responses

Exceptions

- Responses can be separated into two distinct categories:

  - Normal

    - Module has apparently performed satisfactorily

  - Abnormal

    - Module has not performed satisfactorily

  - Abnormal responses may be used to initiate appropriate further fault tolerance activities

  - Abnormal responses are often called exceptional responses or exceptions

  - Exceptions and exception handling can form a framework for implementing fault tolerance

Module B

Service Requests

Service Requests

Abnormal Responses

Abnormal Responses

Normal Responses

Normal Responses

Error Detection and Exception Handling

- Can separate the activity of a module into two parts:

  - Normal

  - Error detection and exception handling

Error Detection and Exception Handling: Cases

A

C

B

E

D

- Case One

  - Error detection check in B detects are error and raises an internal exception

  - Two possibilities:

    - B can correct the error and the exception is masked

    - The exception is propagated to the higher-level module A

- Case Two

  - Call from B to D returns exceptionally

  - Two possibilities

    - B is able to mask the exception and provide proper service to A despite the exceptional return from D

    - The exception must be propagated up to A

Classification of Exceptions: 1

- Useful to be able to classify exceptions

  - Categorise the fault

  - Appropriate fault tolerance response

- First, consider the service request that a component receives

  - A legal request should result in a normal response

  - But if illegal request is received then signal an exception

- Classify this as an interface exception

  - Since it is generated in response to a request which did not conform to the specification of the interface to that component

    - Illegal instruction

    - Divide by zero

    - Access violation

- What caused the invoking component to generate an illegal request?

  - Maybe a design (or other) fault in that component

Classification of Exceptions: 2

- Consider a legal service request:

- But suppose that the component subsequently detects an error

  - An internal exception is raised

- If local exception handlers can deal with the problem

  - The component can return to normal operation

- However, if the component is unable to deal with the problem

  - A failure of the component is looming (due to some fault which cannot be masked)

  - the component should signal a failure exception

- this warns of the potential failure of the component

  - single bit parity check on a memory word

- A component will always signal a failure exception if it cannot provide its specified service

- Suitable located exception handles must strive to deal with all exceptions that are raised

  - Otherwise the exception must be propagated

Normal Activity

Exception Handling

Service Requests

Service Requests

Return to normal

Interface Exceptions

Normal Reponses

Normal Reponses

Interface Exceptions

Failure Exceptions

Failure Exceptions

Internal Exceptions

Idealised Fault Tolerant Component (Module)

- Code Example

Begin F
    Receive request (readblock, i) from P
    Begin_Service
        if i ≠ Legal Block Number then signal INTERFACE_EXCEPTION
        n = MapToDiskBlock(i); Response = ReadFromDisk(n)
        if any exception signalled then raise BLOCK_CORRUPTED
            …
        EXCEPTION_HANDLERS
        When (BLOCK_CORRUPTED)
        {
            Response = ReadFromBackupDisk(n)
            if any exception signalled then signal FAILURE_EXCEPTION1
        }
        When (DIV_BY_ZERO) signal FAILURE_EXCEPTION2
        When (others) signal FAILURE_EXCEPTION0
    End_Service
    Return (Response) to P
End F

- Interface exceptions are signalled from normal activity

- Failure/interface/internal exceptions invoke the exception handling part of the component, which applies fault tolerance techniques

  - If fault tolerance is successful, then the component returns to providing normal service

  - If fault tolerance measures are not successful, then the component must signal a failure exception

Unanticipated Faults

- For anticipated error/fault situations, a corresponding exception handler can often be designed relatively easily

- But now consider an unanticipated fault manifestation in B (eg. a design fault)

  - Case 1:

    - B does not detect an error, and makes a normal response to A, which A may detect subsequently as an unexpected error

  - Case 2:

    - B detects an unexpected error and signals an unexpected failure exception to A

- Now, what can A do (as a general approach)?

  - Propagate: give up and signal an unexpected failure exception up the line

  - Handle: invoke an alternative module B’

- But how can B’ not contain the same design fault?

  - Must be designed and developed differently to B

- Concept of diverse software

  - Common design faults possible but less likely

  - Technique to provide tolerance to software faults